The Islamia University Bahawalpur
Department of Electronic Engineering

Faculty of Engineering

Complex Engineering Problem (CEP)
Artificial Intelligence (COMP-03661)

Submitted to:

Dr. Dileep Kumar

Submitted by:
	Shahroz Chaudhary	(F22BETEN1M01017)

Salamat Ali		(F22BETEN1M01019)

Session:

2022 - 2026

1. Title

IoT-Enabled Edge-AI Smart Lighting Control System

2. Equipment Used

ESP32 microcontroller, LDR sensor, temperature sensor, relay module, TFT LCD display, and 5V DC power adapter.

3. Introduction

This project focuses on the development of an IoT-enabled Edge-AI smart lighting control system designed to optimize energy consumption through intelligent decision-making. By performing machine learning inference directly on the ESP32, the system reduces latency and dependency on cloud resources.

4. Methodology

The proposed system acquires real-time data from light intensity and temperature sensors interfaced with the ESP32 microcontroller. The sensor readings are normalized and processed locally using a trained machine learning model deployed on the device. Based on the predicted lighting condition, the relay module is actuated to control the lighting load. System parameters and operational status are continuously uploaded to the IoT dashboard for monitoring and analysis.

5. Result

Experimental evaluation shows that the Edge-AI-based lighting control system operates reliably under varying environmental conditions. The AI-based approach demonstrates improved accuracy and energy efficiency compared to conventional threshold-based control.

6. Conclusion

The implementation confirms that integrating edge-based machine learning with IoT systems can significantly enhance energy efficiency and system intelligence. The proposed design is suitable for real-world smart lighting applications.

7. System Components and Hardware

| Component Name | Specification | Role in System | Approx. Price |
| --- | --- | --- | --- |
| ESP32 | Dual-core, Wi-Fi | Edge controller | PKR 1400 |
| LDR Sensor | 0–2000 lux | Ambient light sensing | PKR 150 |
| LM-35 | -55–150 °C | Context sensing | PKR 300 |
| Dual Channel Relay Module | 10A, 220V | Lighting control | PKR 400 |
| Zmct103c | 5A | For Power | PKR 375 |
| Oled | 0.96 inch | User feedback | PKR 490 |
| Resistance | 10kohm | Voltage divider | PKR 5 |
|  |  | Total | PKR 3090 |

8. Schematic Diagram and Pin-Level Connectivity

9. Logical Flow and Control Strategy Representation

10. Data Collection and Real-World Scenario Mapping

| Scenario | LDR | Temp | AI Prediction | Light Action | Folder Path |
| --- | --- | --- | --- | --- | --- |
| Early morning (low light) | 300 | 15 °C | Dim (0) | 🔆 Light ON | ….\lidar_test\Dataset\data_morning |
| Late morning (bright) | 1200 | 22 °C | Bright (1) | 💡 OFF | ….\lidar_test\Dataset\after_sunset_20mint.csv |
| Noon (glare sunlight) | 1650 | 28 °C | Glare (2) | 💡 OFF | ….\lidar_test\Dataset\data_noon |
| Cloudy outdoor | 1100 | 19 °C | Bright (1) | 💡 OFF | ….\lidar_test\Dataset\data_noon |
| Indoor lights off | 400 | 17 °C | Dim (0) | 🔆 ON | ….\lidar_test\Dataset\indoornoon |
| Indoor lights on | 900 | 20 °C | Bright (1) | 💡 OFF | ….\lidar_test\Dataset\indoornoon |
| Evening sunset | 450 | 16 °C | Dim (0) | 🔆 ON | ….\lidar_test\Dataset\noondata |
| Night (streetlight) | 280 | 14 °C | Dim (0) | 🔆 ON | ….\lidar_test\Dataset\night_ambient |
| High-temperature test | 1100 | 30 °C | Glare (2) | 💡 OFF | ….\lidar_test\Dataset\sunset_time |
| Fan/cool air test | 950 | 15 °C | Bright (1) | 💡 OFF | ….\lidar_test\Dataset\sunset_time |

11. Machine Learning Model Selection and Tuning

| Model | Accuracy | Precision | Recall | F1-Score | MCC |
| --- | --- | --- | --- | --- | --- |
| Random Forest | 0.91318 | 0.912595 | 0.91318 | 0.912809 | 0.869844 |
| Decision Tree | 0.910031 | 0.909365 | 0.910031 | 0.90953 | 0.865206 |
| Gradient Boosting | 0.907332 | 0.907758 | 0.907332 | 0.907055 | 0.86146 |
| KNN | 0.878093 | 0.877397 | 0.878093 | 0.87771 | 0.817171 |
| AdaBoost | 0.764283 | 0.781364 | 0.764283 | 0.757254 | 0.661563 |
| QDA | 0.724696 | 0.721882 | 0.724696 | 0.713133 | 0.595495 |
| Logistic Regression | 0.7148 | 0.713953 | 0.7148 | 0.713848 | 0.572688 |
| Naive Bayes | 0.71345 | 0.709095 | 0.71345 | 0.702607 | 0.577369 |
| SVM | 0.677463 | 0.75171 | 0.677463 | 0.685494 | 0.54088 |

Confusion Matrix of Random Forest

Confusion Matrix of Light GBM

12. Energy Consumption Comparison

| Mode | Per hour power Consumption | Per Day Power Consumption | Unit consumes in Month |
| --- | --- | --- | --- |
| Threshold | 29.04 Wh | 29.04x11 = 319.44 | 9.583 |
| Ai | 29.04 Wh | 29.04x10 = 290.4 | 8.712 |

13. Model Size and Edge Optimization Analysis

| n_estimators | max_ depth | min_samples_split | min_samples_leaf | Train Accuracy | Test Accuracy | Model Size (KB) |
| --- | --- | --- | --- | --- | --- | --- |
| 50 | Nan | 10 | 2 | 0.923838 | 0.916329 | 576.5869 |
| 50 | 20 | 10 | 2 | 0.923838 | 0.916329 | 576.1943 |
| 100 | Nan | 10 | 2 | 0.924063 | 0.916329 | 1152.382 |
| 75 | Nan | 10 | 2 | 0.923726 | 0.915879 | 864.5781 |
| 50 | 15 | 10 | 4 | 0.922713 | 0.915879 | 513.3213 |
| 111 | 20 | 10 | 2 | 0.923951 | 0.915879 | 1276.437 |
| 100 | 20 | 10 | 2 | 0.923951 | 0.915879 | 1151.958 |
| 111 | Nan | 10 | 2 | 0.923951 | 0.915879 | 1277.691 |
| 50 | 20 | 10 | 1 | 0.925188 | 0.91543 | 591.1699 |

14. IoT Dashboard Data Representation